# Packet Tracer - Use Diagnostic Commands

# Objectives

Part 1: Gather End User Device Settings

Part 2: Gather Information about Network Devices

Part 3: Diagnose Connectivity Issues

# Background / Scenario

In this Packet Tracer (PT) activity, you will use various commands to gather device information and troubleshoot device configuration and connectivity issues. Device information includes IP address, default gateway, and DNS server settings. These settings are critical to enable a device to communicate on networks and connect to the internet.

# Instructions

## Part 1: Gather End User Device Settings

In this part, you will document the IP address settings for end devices.

### Step 1: Document the IP address settings for HQ-Laptop-1.

a.     The activity opens in the HQ cluster. The Wiring Closet is the tall, black chassis in the bottom left corner of the first floor. Locate all the devices on the first floor: PCs 1-1, 1-2, 1-3, and 1-4; printer FL-1P; and HQ-Laptop-1.

b.     Click HQ-Laptop-1 > Desktop tab > Command Prompt.

c.     Enter the ipconfig command.

#### Questions:

Which IPv4 address is displayed for the Wireless0 Connection?

Type your answers here.

If the IPv4 address is in the 169.254.0.0/16 range, what method is being used to assign IPv4 addresses? Why is the laptop assigned an IPv4 address in the 169.254.0.0/16 range?

Type your answers here.

If the IPv4 address is in the 169.254.0.0/16, wait a few seconds and repeat the ipconfig command.

#### Questions:

When the IPv4 address is no longer from 169.254.0.0/16 range, what is the IP addressing information displayed? Record your answers in the table below.

| Wireless0 | IP Addressing Information |
| --- | --- |
| Link-local IPv6 Address |  |
| IPv6 Address |  |
| IPv4 Address |  |
| Subnet Mask |  |
| Default Gateway |  |
| DNS Servers |  |

Blank Line, No additional information

Blank Line, No additional information

Do you see a DNS server address? Explain.

Type your answers here.

d.     Enter the ipconfig /all command.

#### Question:

Do you see the DNS server address? What is it?

Type your answers here.

### Step 2: Document the IP address settings for Net-Admin.

a.     Click Wiring Closet > Net-Admin > Desktop tab > Command Prompt.

b.     Enter the ipconfig /all command.

#### Question:

What is the IP addressing information displayed under the FastEthernet0 interface? Record your answers in the table below.

| FastEthernet0 | IP Addressing Information |
| --- | --- |
| Physical Address |  |
| Link-local IPv6 Address |  |
| IPv6 Address |  |
| IPv4 Address |  |
| Subnet Mask |  |
| Default Gateway |  |
| DNS Servers |  |

Blank Line, No additional information

Blank Line, No additional information

## Part 2: Gather Information about Network Devices

In this part, you will document information about the link to ISP. You will then document the IP addressing information for all the end devices in HQ and discover that devices belong to different virtual local area networks (VLANs).

### Step 1: Gather network connection information about the link between HQ and ISP.

The HQ-Edge router is the router between the HQ network and the ISP. We need to identify the upstream device information located in the ISP.

a.     In the Wiring Closet left rack, click HQ-Edge > CLI tab.

b.     Press Enter to get the HQ-Edge> prompt, and then enter the enable command.

c.     Enter the show ip route | begin Gateway command.

#### Questions:

What is the address for the gateway of last resort (or default gateway)?

Type your answers here.

Why is the next hop address not displayed?

Type your answers here.

d.     Enter the show running-config | begin ip route command.

#### Question:

How is the default route configured? Does it use the next hop address?

Type your answers here.

e.     Enter the show cdp neighbors detail command.

#### Questions:

What is the IPv4 address of the next hop (ISP) address?

Type your answers here.

Which port on the ISP router is connected to HQ-Edge?

Type your answers here.

What IOS version is used on the ISP router?

Type your answers here.

f.      Enter the ping 10.0.0.49 command.

g.     Enter the show arp command.

#### Question:

What is the MAC address of the interface on the ISP router that is connected to HQ-Edge?

Type your answers here.

h.     Close HQ-Edge and exit the Wiring Closet.

### Step 2: Gather network connection information about the devices in HQ.

a.     From 1-1, 1-2, 1-3, 1-4, FL-1P, and HQ-Laptop-1, use the ipconfig command to find their IPv4 addresses and Default Gateways.

| Device | IPv4 Address | Default Gateway |
| --- | --- | --- |
| 1-1 |  |  |
| 1-2 |  |  |
| 1-3 |  |  |
| 1-4 |  |  |
| FL-1P |  |  |
| HQ-Laptop-1 |  |  |

Blank Line, No additional information

Blank Line, No additional information

b.     From PC 1-1, open Command Prompt, and then enter the arp -a command.

#### Question:

What information is displayed?

Type your answers here.

c.     Use the ping command to ping 1-2, 1-3, 1-4, FL-1P, and HQ-Laptop-1.

d.     Enter the arp -a command.

#### Questions:

What information is displayed?

Type your answers here.

Why do the entries in the ARP table not contain information about devices in the 192.168.20.0 and 192.168.50.0 networks while the ping is successful?

Type your answers here.

e.     To find the route a packet takes to reach the DNS server, enter the tracert 10.2.0.125 command.

#### Question:

What information is displayed?

Type your answers here.

How many routers, or hops, are between PC 1-1 and the DNS server?

Type your answers here.

## Part 3: Diagnose Connectivity Issues

In this part, you will use a variety of diagnostic commands and techniques. You will use the nslookup command to query a DNS server and troubleshoot a DNS database. You will then diagnose why a ping fails but web access is successful. Finally, you will use the netstat command to discover which ports are listening on the target device.

### Step 1: Test a URL to investigate a connectivity issue.

a.     On PC 1-1, close the Command Prompt, and then click Web Browser.

b.     Enter the URL test.ptsecurity.com.

#### Question:

Does the web page display? If not, what is the message?

Type your answers here.

c.     Enter the IP address 192.168.75.2.

#### Questions:

Does the web page display?

Type your answers here.

Why does the web page display by using the IP address but not the domain name?

Type your answers here.

### Step 2: Use the nslookup command to verify DNS service.

a.     Close Web Browser, and then click Command Prompt.

b.     Enter the ping test.ptsecurity.com command.

#### Questions:

What message is displayed?

Type your answers here.

What does the message indicate?

Type your answers here.

c.     Enter the nslookup test.ptsecurity.com command.

#### Question:

What message is displayed?

Type your answers here.

Which server is the default DNS server?

Type your answers here.

d.     The nslookup command supports the use of alternate DNS server. Enter the nslookup /? command to learn options available for the command.

e.     Enter the nslookup test.ptsecurity.com 192.168.99.3 command and press Enter.

Note: Packet Tracer may take several seconds to converge.

#### Questions:

What message is displayed?

Type your answers here.

In Step 2c, why is the domain name unable to be resolved?

Type your answers here.

### Step 3: Use output from the ping command to diagnose connectivity issues.

a.     Enter the ping mail.cybercloud.com command.

#### Questions:

What message is displayed?

Type your answers here.

What information is indicated by the message?

Type your answers here.

b.     Enter the ping www.ptsecurity.com command.

#### Questions:

What message is displayed?

Type your answers here.

What information is indicated by the message?

Type your answers here.

c.     Close the Command Prompt, open Web Browser, and then navigate to www.ptsecurity.com.

#### Questions:

Does the web page display?

Type your answers here.

What conclusion can be drawn?

Type your answers here.

### Step 4: Use the netstat command to find active and listening ports.

a.     Close Web Browser and reopen Command Prompt.

b.     In HQ, click the Wiring Closet

c.     From the right track, click the FTP server > Desktop tab > Command Prompt.

d.     Arrange the PC 1-1 and FTP server Command Prompt windows side by side.

e.     From the PC 1-1 window, enter the netstat command.

#### Questions:

What message is displayed? Does it show any data?

Type your answers here.

f.      From the FTP server, enter the netstat command.

#### Question:

What message is displayed? Does it show any data?

Type your answers here.

g.     On FTP server, enter the ipconfig command to determine its IP address.

h.     From PC 1-1, start an FTP session with the FTP server.

i.      On the FTP server, enter the netstat command.

#### Questions:

What message is displayed? Is there any new information?

Type your answers here.

Which port is the listening port and what is the status of the connection?

Type your answers here.

j.      From PC 1-1, enter bob as the username.

k.     From the FTP server, enter the netstat command.

#### Question:

Does the displayed information change?

Type your answers here.

l.      From PC 1-1, enter cisco123 as the password.

m.   From PC 1-1, enter the dir command.

n.     From the FTP server, enter the netstat command.

#### Questions:

Does the displayed information change?

Type your answers here.

What is indicated by this new entry?

Type your answers here.

o.     From PC 1-1, enter the Sample2.txt command and press Enter. This will upload the Sample2.txt file to the FTP server.

p.     From the FTP server, enter the netstat command.

#### Questions:

Does the displayed information change?

Type your answers here.

q.     Wait for a few seconds and then enter the netstat command again.

#### Question:

Does the displayed information change?

Type your answers here.

r.      From PC 1-1, enter the quit command.

s.     From the FTP server, enter the netstat command.

#### Question:

Does the displayed information change?

Type your answers here.

t.      From PC 1-1, close Command Prompt, and then open Web Browser.

u.     Navigate to 192.168.75.2.

v.     From the FTP server, enter the netstat command.

#### Questions:

Does the displayed information change?

Type your answers here.

What does this new entry indicate?

Type your answers here.

End of document