# Packet Tracer - Investigate Disaster Recovery

# Objectives

Part 1: Review a Switch Configuration

Part 2: Backup Files to a TFTP Server

Part 3: Replace a Failed Switch

Part 4: Restore Network Operations

# Background / Scenario

In this Packet Tracer (PT) activity, you will back up switch configuration files, replace a failed switch with a new switch, and then restore network operations by applying the backed up configuration from the failed switch to the new switch. The backup configuration files are saved to a Trivial File Transfer Protocol (TFTP) server. You are required to restore the saved files from the TFTP server to get the replacement switch online with as little down time as possible.

Note: The activity opens in the Wiring Closet for HQ. Although you can navigate out of the Wiring Closet, all tasks in this activity will occur inside the Wiring Closet. Switching to Logical mode is disabled.

# Instructions

## Part 1: Review a Switch Configuration

In this part, you will review and document the current configuration of the MDF-1 switch in the HQ Wiring Closet. This information will be necessary for manually configuring a replacement switch and verifying the new switch is operating as expected.

### Step 1: Observe the contents of NVRAM.

a.     Click MDF-1 > CLI tab, and then press Enter.

b.     Enter the enable command, and then enter the dir nvram command to observe the contents of NVRAM.

#### Question:

What is the size of the startup-config file?

Type your answers here.

### Step 2: Document the VLANs and other important configuration information.

a.     Enter the show vlan command.

#### Question:

What VLANs have been configured on MDF-1?

Type your answers here.

b.     Enter the show run command. Review the output to document the following information, which you will need to manually configure on a switch after a disaster.

#### Question:

Record the following settings in the following table:

| MDF-1 Settings | Command Output |
| --- | --- |
| VLAN 99 IP Address |  |
| Default Gateway IP Address |  |
| VLAN Assignment of Interface F0/1 |  |
| Native LAN and Trunk Status of G0/1 |  |

Blank Line, No additional information

Blank Line, No additional information

## Part 2: Backup Files to a TFTP Server

In this part, you will copy the configuration files for the MDF-1 switch to the TFTP server. You will then verify that the files are listed on the TFTP server.

### Step 1: Enable the TFTP service on the FTP server.

a.     In the Wiring Closet, on the right track, click the FTP server > Desktop tab > Command Prompt.

b.     Enter the ipconfig command.

#### Question:

What is the IP address for the FTP server?

Type your answers here.

c.     Click the Services tab, and then under SERVICES, click TFTP.

d.     Enable the TFTP service.

### Step 2: Upload the vlan.dat and the startup-config files to the TFTP server.

a.     Click MDF-1, and then CLI tab, if necessary. If you were logged out, enter the enable command again.

b.     Enter copy flash tftp command and specify vlan.dat as the source filename. You documented the IP address in the previous step. Enter MDF-1_vlan.dat for the destination filename.

#### Question:

Record the command below:

Type your answers here.

c.     Enter the copy startup-config tftp command to copy the configuration to the TFTP server. You documented the IP address in the previous step. Enter MDF-1_startup-config as the destination filename.

#### Question:

Record the command below:

Type your answers here.

### Step 3: Verify that the files are on the TFTP server.

Click FTP server. Under TFTP in SERVICES, verify the two files are listed in the File section. If necessary, refresh the File list by clicking another service and then clicking the TFTP service again.

## Part 3: Replace a Failed Switch

Assume that the MDF-1 switch has failed. This could be from a power surge, a corrupted chip, or some other environmental hazard or hardware failure. In this Part, you will install a replacement switch and move the cable connections from the failed switch to the new switch.

### Step 1: Add a new switch to the network.

a.     On the Table in the Wiring Closet, locate spare-switch_01.

b.     Click and drag it to the rack below HQ-WLC-1.

c.     Click spare-switch_01 > CLI tab, and then press Enter.

d.     Enter the following commands to deactivate all the interfaces.

enable

configure terminal

interface range f0/1 - 23, g0/1 - 2

shutdown

exit

### Step 2: Move the cable connections from the MDF-1 switch to new switch.

a.     On the top toolbar, click Zoom In several times until you can easily see the cable connections for both MDF-1 and spare-switch_01.

Alternatively, you can right click each switch and choose Inspect Front. But you will need to do this each time you move a connection from MDF-1 to spare-switch_01.

b.     Click and drag a cable connection from MDF-1 to the same port number on spare-switch_01. Repeat until all cables are moved from MDF-1 to spare-switch_01.

c.     To verify cables are in the correct ports, right click spare-switch_01 and choose Inspect Front. Zoom in and then float your mouse of each cable, wait for the information popup, and then make sure the cable connections map to this table.

| MDF-1 Interface Port | Connected Device |
| --- | --- |
| F0/1 | FTP Server |
| F0/2 | MAIL Server |
| F0/3 | AAA-RADIUS Server |
| F0/15 | Net-Admin PC |
| F0/19 | FL-1 F0/19 |
| F0/20 | FL-1 F0/20 |
| F0/21 | FL-2 F0/21 |
| F0/22 | FL-2 F0/22 |
| G0/1 | HQ Edge Router |

Blank Line, No additional information

d.     Right click the Rack and choose Manage All Cables on Rack.

e.     Uninstall MDF-1 from the Rack. Click and drag it to the Table.

f.      On the top toolbar, click Zoom Reset.

## Part 4: Restore Network Operations

In this Part, you will manually configure the new switch so that it can access the TFTP server. You will then copy the configuration files from the TFTP server to the new switch and verify the switch is operating as expected.

### Step 1: Configure spare-switch_01 to access the network.

To access the TFTP server over the network, the spare switch will need network information configured manually. Enter the following configuration into spare-switch_01 to connect it to the network and prepare it for TFTP server access.

vlan 99

name Admin

exit

interface vlan 99

ip address 192.168.99.150 255.255.255.0

exit

ip default-gateway 192.168.99.1

interface fa0/1

switchport mode access

switchport access vlan 75

no shutdown

interface g0/1

switchport mode trunk

switchport trunk native vlan 99

no shutdown

end

### Step 2: Test connectivity to the TFTP server.

Enter ping 192.168.75.2 to verify spare-switch_01 can access the TFTP server.

### Step 3: Download the vlan.dat and startup-config files from the TFTP server.

a.     Enter the copy tftp flash command. Specify the IP address of the TFTP server. The source filename is MDF-1_vlan.dat. The destination filename MUST be vlan.dat. Confirm you want to overwrite the current vlan.dat file.

#### Question:

Record the command below:

Type your answers here.

b.     Enter the dir flash command to verify the vlan.dat file is in the directory.

#### Question:

Record the command below:

Type your answers here.

c.     Enter the copy tftp startup-config command. Specify the IP address of the TFTP server. The source filename is MDF-1_startup-config. The destination filename MUST be startup-config.

#### Question:

Record the command below:

Type your answers here.

d.     Enter the dir nvram command to verify the startup-config file is now in NVRAM.

#### Question:

Record the command below:

Type your answers here.

What is the size of the startup-config file?

Type your answers here.

Is this the same size as the startup-config recorded in Part 1, Step 1?

Type your answers here.

### Step 4: Reload and verify the new switch now has the correct configuration.

a.     Enter the reload command. The startup-config file will be copied into RAM and become the running configuration.

Important: Answer no to the prompt, System configuration has been modified. Save?, and then press Enter to confirm reload.

#### Question:

Record the command below:

Type your answers here.

b.     After the switch reloads, review the configuration.

o    The hostname is now be MDF-1.

o    Enter the show vlan command and verify VLANs you documented in Part 1, Step 2 are listed.

o    Enter the show ip interface brief command. Verify that your connected physical ports are now all up.

End of document