# Lab - Troubleshoot Using Network Utilities

# Objectives

●   Interpret the output of commonly used network command line utilities.

●   Determine which network utility can provide the necessary information to perform troubleshooting activities in a bottom-up troubleshooting strategy.

# Background/Scenario

There are a number of problems that can cause networking connectivity issues. In this lab, you will use network utilities that can help you to identify connectivity issues in wireless networks. The network command line utilities are also useful to detect problems in a wired network.

# Required Resources

●   1 PC (Windows 10 with a wired and wireless NIC installed)

●   A wireless router

●   Internet connectivity

# Instructions

## Part 1: Network Connections

### Step 1: Connect to a wireless network.

a.     Disconnect the Ethernet cable from your computer. An “orange triangle” appears over the Connections icon in the system tray.

b.     Click the Connection icon in the system tray.

#### Question:

What is the name of an available wireless connection?

Type your answers here.

c.     Click one of the available wireless connections. Connect to the network. Enter login information if required. Confirm that the connection is successful.

d.     Confirm that the connection is successful.

### Step 2: Verify that the network adapter is operational.

When a connectivity problem is reported, the first step in a bottom-up troubleshooting strategy is to determine whether the NIC and the operating system settings on the computer are functioning correctly.

a.     Open the Control Panel, select Network and Sharing Center. Right-click Start and select Control Panel. Click Network and Sharing Center. Click Change adapter settings.

b.     Select the Wireless Network Connection. Right click the adapter and select Status from the menu. If the Status choice is grayed out, it indicates that the adapter is either not enabled or not connected to a wireless SSID.

c.     In the status window, verify that the connection is enabled and that the connection SSID is correct. Click Details to open the adapter details window.

d.     The Details window shows the current IP configuration active on the network adapter. It displays both the IPv4 and IPv6 configurations. If DHCP is active, the lease information is shown.

#### Questions:

Is DHCP enabled on the PC?

Type your answers here.

When does the DHCP lease expire?

Type your answers here.

### Step 3: Confirm correct network configuration.

a.     Open a Command Prompt.

b.     Enter ping 127.0.0.1. The IP address 127.0.0.1 is also referred to as the localhost address. A successful ping to the localhost address indicates that the TCP/IP protocol stack is operational on the computer. If the localhost address does not reply to a ping command, there might be an issue with the device driver or the network interface card.

#### Question:

Was the ping command successful?

Type your answers here.

c.     Use the ipconfig command. Identify the IP address, subnet mask and default gateway addresses configured on the computer.

If the local IPv4 address is a host address on the 169.254.0.0/16 network, the computer received its IP address configuration through the Automatic Private IP Addressing (APIPA) feature of the Windows operating system.

#### Question:

What problems can cause a computer to receive an APIPA address?

Type your answers here.

If the computer is assigned an APIPA address, there might be an issue with the DHCP server. If the wireless router is providing the DHCP services, confirm that the DHCP service is configured correctly and that the IP address range is large enough to accommodate all the devices that may attach wirelessly.

#### Question:

What is the IP address of the default gateway assigned to your PC?

Type your answers here.

d.     To test if the PC can reach the default gateway through the network, ping the default gateway IP address.

A successful ping indicates that there is a connection between the computer and the default gateway.

If the ping command does not complete successfully, make sure that the IP address of the gateway is typed correctly and that the wireless connection is active.

e.     Type net view. The net view command, when issued on a Windows PC, displays the computer names of other Windows devices in your Windows domain or workgroup. When net view displays the names of other computers it indicates that your computer can successfully send messages across the network.

C:\Users\Student> net view

#### Question:

List the computer names that are displayed.

Type your answers here.

Note: Depending on the configuration of the PCs in your lab, net view may not return any computer names or may display an error message. If this is the case, move on to the next step.

## Part 2: External Connectivity

### Step 1: Test external connectivity.

If you have an external connection, use the following methods to verify the operation of the default gateway and the DNS service.

Note: Your output may be varied.

a.     The Windows tracert command performs the same function as the traceroute command used within the Cisco IOS. Use the tracert command along with your school’s Web site URL or the Cisco Networking Academy Web site. For example, enter tracert www.netacad.com.

C:\Users\Student> tracert www.netacad.com

Tracing route to Liferay-Prod-1009279580.us.-east-1.elb.amazonaws.com [52.5.233.103]

over a maximum of 30 hops:

1 1 ms 57 ms 3 ms 192.168.1.1

2 * 12 ms 12 ms 10.39.176.1

3 14 ms 28 ms 11 ms 100.127.65.248

4 10 ms 26 ms 21 ms 70.169.73.90

5 35 ms 32 ms 36 ms 68.1.2.109

<output omitted>

Trace complete.

The tracert command displays the path that the packet takes between the source and destination IP addresses. Each router that the packet travels through to reach the destination address is shown as a hop in the tracert output. If there is a network issue on the path, the tracert output will stop after the last successful hop. The first hop in the output is the default gateway of the source PC, the last entry will be the destination address when the tracert command completes successfully.

b.     The command tracert uses the configured DNS server to resolve the fully qualified domain name to an IP address before beginning to trace the router to the destination. Using tracert or ping with a domain name instead of an IP address can confirm that the DNS server is providing name resolution services.

#### Questions:

What IP address was returned by the DNS server?

Type your answers here.

What would happen if the DNS server could not resolve the domain name of the server?

Type your answers here.

c.     Use the nslookup command with the IP address you just discovered. The nslookup command is a utility that can be used to troubleshoot DNS problems.

Type nslookup 72.163.6.233. The IP address in this example is assigned to a server at Cisco Systems.

#### Questions:

What domain name was returned?

Type your answers here.

What DNS server did the nslookup command use to resolve the domain name?

Type your answers here.

Does the DNS server IP address match the one displayed in the ipconfig /all output?

Type your answers here.

d.     When the configured DNS server cannot resolve domain names or IP addresses, it is possible to set nslookup to try to resolve the names using a different DNS server. If another DNS server can resolve the addresses, but the configured DNS server cannot, there could be a problem with the DNS server configuration. Enter nslookup /? to view the options that can be used to test and troubleshoot DNS issues.

C:\Users\Student> nslookup /?

Usage:

nslookup [-opt ...] # interactive mode using default server

nslookup [-opt ...] - server # interactive mode using 'server'

nslookup [-opt ...] host # just look up 'host' using default server

nslookup [-opt ...] host server # just look up 'host' using 'server'

### Step 2: Test Application layer connectivity.

Open a web browser. Enter www.cisco.com in the Address field.

#### Question:

Does the Cisco.com web page load in the browser? What underlying network functions have to be working for the web page load?

Type your answers here.

# Reflection

1.     The steps in this lab represent a bottom-up troubleshooting strategy, where the effort starts with the OSI model physical layer and finishes with verifying the functionality of the Application layer. What are the other two troubleshooting strategies used by network technicians to isolate problems?

Type your answers here.

2.     Which strategy would try first when presented with a network connectivity problem? Explain.

Type your answers here.